SNP 6230RH 2MEGAPIXEL FULL HD 23X NETWORK IR PTZ DOME CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer. The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

2.02	SNP 6230RH 2MEGAPIXEL FULL HD 23X NETWORK IR PTZ DOME CAMERA

- These cameras shall offer Max. 2M (1920 x 1080) resolution and 16:9 Full HD (1080p) resolutions.

- The camera shall provide 4.4 ~ 101.2mm (23x) IR corrected optical zoom, and 16x digital zoom.

- The camera shall support H.264, MJPEG dual codec, and multiple streaming.

- The camera shall feature Day & Night (ICR).

- The camera shall feature Auto tracking and intelligent video analytics.

- The camera shall feature Wide Dynamic Range with a gain of 120dB.

- The camera shall support SD/SDHC/SDXC memory slot.

- The camera shall support Bi-directional audio.

- The camera has IP66 and IK10 certifications.

- The camera has IR LED (2ea).

2.03	VIDEO

| A. Imaging Device |  | 1/2.8" 2.38M CMOS |
| --- | --- | --- |
| B. Total Pixels |  | 1,952(H) x 1,116(V), 2.18M pixels |
| C. Effective Pixels |  | 1,944(H) x 1,104(V), 2.14M pixels |
| D. Scanning System |  | Progressive |
| E. Min. Illumination |  |  |
|  | 1. Color | 0.3Lux (1/30sec, F1.6, 50IRE), 0.005Lux (2sec, F1.6, 50IRE) 0.1Lux (1/30sec, F1.6, 30IRE), 0.0005Lux (2sec, F1.6, 30IRE) |
|  | 2. B/W | 0Lux (IR LED on) |
| F. S/N Ratio |  | 50dB |
| G. Video Output |  | CVBS : 1.0 Vpp / 75Ω composite, 720 x 480(N), 720 x 576(P), for installation |

2.04	LENS

| A. Focal Length (Zoom Ratio) | 4.4 ~ 101.2mm (Optical 23x) |
| --- | --- |
| B. Max. Aperture Ratio | F1.6(Wide) / F4.5(Tele) |
| C. Angular Field of View | H : 63.68˚(Wide) ~ 3.1˚(Tele) V : 37.52˚(Wide) ~ 1.76˚(Tele) |
| D. Min. Object Distance | 1.5m (4.92ft) |
| E. Focus Control | AF / One-shot AF / Manual |
| F. Lens Type | DC auto iris (IR corrected) |
| G, Mount Type | Board-in type |

2.05	PAN/TILT/ROTATE

| A. Pan Range | 360˚ Endless |
| --- | --- |
| B. Tilt Range | 190˚ (-5˚ ~185˚) |
| C. Pan/Tilt Speed | Preset : 250˚/sec, Manual : 0.024˚/sec ~ 120˚/sec |
| D, Sequence Accuracy | Preset (255ea), Swing, Group (6ea), Trace, Tour, Auto run, Schedule |
| E. Preset Accuracy | ±0.2 |
| F. Azimuth | Yes (E / W / S / N / NE / SE / NW / SW OSD) |
| G. Auto Tracking | Off/On |

2.06	OPERATION

| - IR LED | 2ea |
| --- | --- |
| - Viewable Length | 100m (328.08ft) |
| - Camera Title | Off / On (Displayed up to 15 characters) |
| - Day & Night | Auto (ICR) / Color / B/W |
| - Backlight Compensation | Off / BLC / HLC / WDR |
| - Wide Dynamic Range | 120dB |
| - Contrast Enhancement | SSDR (Samsung Super Dynamic Range) (Off / On) |
| - Digital Noise Reduction | SSNRIII (2D+3D noise filter) (Off / On) |
| - Digital Image Stabilization | Off / On |
| - Defog | Off / Auto / Manual |
| - Motion Detection | Off / On (4ea, Rectangular zones) |
| - Privacy Masking | Off / On (32ea, Rectangular zones) - Color : Grey / Green / Red / Blue / Black / White, - Zoom ratio option for mask mode |
| - Gain Control | Off / Low / Medium / High / Manual |
| - White Balance | ATW / AWC / Manual / Indoor / Outdoor / Mercury / Sodium |
| - Electronic Shutter Speed | Minimum / Maximum / Anti flicker (2 ~ 1/12,000sec) |
| - Digital Zoom | 16x, area zoom function support digital zoom 2x |
| - Flip / Mirror | Off / On |
| - Intelligent Video Analytics | Tampering (Scene change), Virtual line, Enter / Exit, (Dis)Appear, Audio detection, Face detection |
| - Alarm I/O | Input 4ea / Output 3ea (Relay type) |
| - Remote Control Interface | RS-485/422 |
| - RS-485 Protocol | Samsung-T/E, Pelco-P/D, Panasonic, Honeywell, AD, Vicon, Bosch, GE |
| - Alarm Triggers | Alarm input, Motion detection, Intelligent video analytics, Network disconnect |
| - Alarm Events | File upload via FTP and E-mail Notification via E-mail Local storage (SD/SDHC/SDXC) or NAS recording at event triggers External output PTZ preset |

2.07	NETWORK

| A. Ethernet |  | RJ-45 (10/100BASE-T) |
| --- | --- | --- |
| B. Video Compression Format |  | H.264 (MPEG-4 Part 10/AVC) : Main / Baseline / High, MJPEG |
| C. Resolution |  | 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768, 800 x 600, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| D. Max. Framerate |  |  |
|  | 1. H.264 | Max. 60fps @ all resolutions |
|  | 2. MJPEG | Max. 15fps @ 1920 x 1080, 1280 x 1024, 1280 x 960, 1280 x 720, 1024 x 768 Max. 30fps @ 800 x 600, 720 x 576, 640 x 480, 640 x 360, 320 x 240, 320 x 180 |
| E. Smart Codec |  | Manual mode (Area-based : 5ea), Face detection mode |
| F. Video Quality Adjustment |  |  |
|  | 1. H.264 | Compression level, Target bitrate level control |
|  | 2. MJPEG | Quality level control |
| G. Bitrate Control Method |  |  |
|  | 1. H.264 | CBR or VBR |
|  | 2. MJPEG | VBR |
| H. Streaming Capability |  | Multiple streaming (Up to 10 profiles) |
| I. Audio In |  | Selectable (Mic in / Line in), Supply voltage : 2.5V DC (4mA), Input impedance : approx. 2K Ohm |
| J. Audio Out |  | Line out (3.5mm stereo mini jack), Max output level : 1 Vrms |
| K. Audio Compression Format |  | G.711 u-law/G.726 selectable, G.726 (ADPCM) 8KHz, G.711 8KHz, G.726 : 16Kbps, 24Kbps, 32Kbps, 40Kbps |
| L. Audio Communication |  | Bi-directional audio (2-way) |
| M. IP |  | IPv4, IPv6 |
| N. Protocol |  | TCP/IP, UDP/IP, RTP(UDP), RTP(TCP), RTCP, RTSP, NTP, HTTP, HTTPS, SSL, DHCP, PPPoE, FTP, SMTP, ICMP, IGMP, SNMPv1/v2c/v3(MIB-2), ARP, DNS, DDNS, QoS, PIM-SM, UPnP, Bonjour |
| O. Security |  | HTTPS(SSL) login authentication, Digest login authentication IP address filtering, User access log, 802.1x authentication (EAP-TLS, EAP-LEAP) |
| P. Streaming Method |  | Unicast / Multicast |
| Q. Max. User Access |  | 15 users at unicast mode |
| R. Edge Storage |  | SD/SDHC/SDXC (Up to 128G) Motion Images recorded in the SD/SDHC/SDXC memory card can be downloaded NAS (Network Attached Storage), Local PC for instant recording |
| S. Application Programming interface |  | ONVIF profile S, SUNAPI (HTTP API), SVNP 1.2, Samsung Techwin Open Platform |
| T. Webpage Language |  | English, French, German, Spanish, Italian, Chinese, Korean, Russian, Japanese, Swedish, Danish, Portuguese, Turkish, Polish, Czech, Rumanian, Serbian, Dutch, Croatian, Hungarian, Greek, Finnish, Norwegian |
| U. Web Viewer |  |  |
|  | 1. Supported OS | Windows XP / VISTA / 7 / 8, MAC OS X 10.7 |
|  | 2. Supported Browser | Microsoft Internet Explorer (Ver.8 ~ 11), Mozilla Firefox (Ver. 9 ~ 19), Google Chrome (Ver. 15 ~ 25), Apple Safari (Ver. 6.0.2(Mac OS X 10.8, 10.7 only), 5.1.7) * Mac OS X only |
| V Central Management Software |  | SmartViewer, SSM |

2.08	ELECTRICAL

| A. Voltage | 24V AC only / 6A |
| --- | --- |
| B. Power Consumption | Max. 30W (Heater off) Max. 35W/90W (Heater off/on, IR on) |

2.09	ENVIRONMENTAL SPECIFICATIONS

| A. Operating Temperature / Humidity | -50°C ~ +55°C (-58°F ~ +131°F) / Less than 90% RH |
| --- | --- |
| B. Storage Temperature / Humidity | -30°C ~ +60°C (-22°F ~ +140°F) / Less than 90% RH |
| C. Ingress Protection | IP66 |
| D. Vandal Resistance | IK10 |

2.10	PHYSCIAL SPECIFICATIONS

| A. Dimension | Ø248.0 x 399.5mm (Ø9.76" x 15.73") |
| --- | --- |
| B. Weight | 7.1Kg (15.65 lb) |
| C. Color/Material | Ivory / Aluminum |

2.11	CERTIFICATIONS

A. CE mark

B. FCC mark

C. UL mark

2.12	WARRANTY

A. 3 years, parts and labor.