Hanwha Vision is a global leading supplier of solutions for IP and analog video surveillance. Building on the company's history of innovation, Hanwha Vision is dedicated to providing systems solutions with the highest levels of performance, reliability and cost-effectiveness. Hanwha Vision is committed to the continued development of innovative systems products for professional security applications.

For additional information, https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: Hanwha Vision (https://www.hanwhavision.com/en/, https://hanwhavisionamerica.com)

    - Model: XNP-C6403

    - Alternates: None

  - GENERAL DESCRIPTION

    - Video Compression and Transmission – The camera shall have the following properties relating to the video signals it produces.

      - H.265, H.264 and MJPEG compression, each derived from a dedicated encoder and capable of being streamed independently and simultaneously

        - H.265/H.264: Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG: Max. 30fps/25fps(60Hz/50Hz)

      - The camera shall be able to configure various resolution selections.

        - 16:9 aspect ratio: 1920 x 1080, 1280 x 720, 800 x 448, 640 x 360

        - 4:3 aspect ratio: 1280 x 960, 1024 x 768, 800 x 600, 640 x 480, 320x240

        - 5:4 aspect ratio: 1280 x 1024, 720 x 576

        - 3:2 aspect ratio: 720 x 480

      - The camera shall support unicast video streaming up to 20 users

      - The camera shall support multicast video streaming up to 128 users

      - The camera shall support multiple video streaming up to 10 profiles.

      - The camera shall be able to configure Dynamic DNS (DDNS). DDNS shall be provided with no additional cost by the manufacturer.

      - The camera shall provide WiseStream Ⅱ, Dynamic GOV and Dynamic FPS to efficiently manage bit rate of the video stream and reduce storage.

    - Camera – The camera device shall have the following physical and performance properties:

      - IP66, IK10, NEMA4X, NEMA-TS 2(2.2.8, 2.2.9) rated for protection against impacts.

      - True day/night operation with IR cut filter. Low light level operation to 0.05 lux at F1.6 in color mode and 0.005 lux at F1.6 in black and white mode.

      - The camera shall be able to produce clear images in highly contrast scenes with multi-exposure wide dynamic range up to 150dB

      - The camera shall support digital noise reduction using both 2D and 3D noise reduction technology.

      - The camera shall be able to configure 32 privacy masking regions with quadrangle.

      - The camera shall provide video display on smart phone (iPhone, Android) to adjust viewing angle, pan, tilt, rotation and focus.

    - Intelligence and Analytics – The camera shall have a suite of intelligent analytic functions.

      - Analytics events based on AI engine

        - Object detection: Person/Face/Vehicle/License plate

        - Attributes: Vehicle(Type: car/bus/truck/motorcycle/bicycle)

        - Support DetectionShot

        - IVA: Virtual line(Crossing/Direction), Virtual area(Loitering/Intrusion/Enter/Exit)

      - Analytics events: Defocus detection, Motion detection, Tampering, Fog detection, Shock detection, Virtual area(Appear/Disappear)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Audio Detection and classification of the following sound with NW I/O Box (External Device)

          - Scream

          - Gunshot

          - Explosion

          - Crashing glass

      - Object auto tracking(Person/Vehicle), Target lock tracking based on AI engine

    - Interoperability – The camera shall be ONVIF Profile S / G and T compliant.

    - The camera shall possess the following further characteristics:

      - Micro SD/SDHC/SDXC memory card with configurable pre-alarm and post-alarm recording intervals

      - NAS recording option with configurable pre-alarm and post-alarm recording intervals

      - Alarms and notifications

        - alarm notification triggers

          - Analytics

          - Network disconnect

          - Alarm input with NW I/O Box

        - available notification means upon trigger:

          - File Upload via FTP and E-mail

          - Notification via E-mail

          - Local storage (SD / SDHC / SDXC) or NAS recording at event triggers

          - Alarm output with NW I/O Box

          - PTZ preset

          - Handover

      - This device has been verified using STP cable. The use of appropriate GND grounding and STP cable is recommended to effectively protect your product and property from transient voltage, thunder stroke, communication interruption.

      - NDAA Compliant

  - DETAILED SPECIFICATIONS

    - Video

      - Sensor: 1/2.8" CMOS

      - scanning: progressive

      - Minimum illumination

        - Color mode: 0.05Lux (1/30sec, F1.6, 30IRE)

        - Black & white mode: 0.005Lux (1/30sec, F1.6, 30IRE)

    - Lens

      - Focal length: 4.25~170mm(40x) zoom (digital 32x, total 1280x zoom)

      - Aperture Ratio: F1.6(Wide) ~ F4.95(Tele)

      - Angular Field of View

        - H: 65.66°(Wide)~1.88°(Tele)

        - V: 39.40°(Wide)~1.09°(Tele)

      - Min. Object Distance: 5m(16.4ft)

      - Focus control: One Shot AF, Focus save

      - Lens Type: DC auto iris

    - Pan / Tilt

      - Pan Range: 360° Endless

      - Pan Speed: Max. 700°/sec, Manual 0.024°/sec~250°/sec

      - Tilt Range: 110°(-20°~90°)

      - Tilt Speed: Max. 500°/sec, Manual 0.024°/sec~250°/sec

      - Sequence: Preset(300ea), Swing, Group(6ea), Trace, Tour, Auto Run, Schedule

      - Preset Accuracy: Up to ±0.1°, Pan / Tilt correction

    - Operational Functions

      - Camera Title: Displayed up to 85 characters

      - Direction Indicator: Support

      - Day/night: Auto(ICR) /Color /BW /Schedule

      - Backlight Compensation: BLC /HLC /WDR(150dB) /SSDR

      - Digital Noise Reduction: SSNRⅤ

      - Digital Image Stabilization: Support(built-in gyro sensor)

      - Defog: Support

      - Motion Detection: 8ea, 8point polygonal zones

      - Privacy Masking: 32ea, Quadrangle Support

      - Gain Control: Manual / Max

      - White Balance: ATW /Narrow ATW /AWC /Manual /Indoor /Outdoor /Mercury /Sodium

      - Electronic Shutter Speed: Minimum / Maximum / Anti flicker (2~1/12,000sec)

      - Video Rotation: Flip, Mirror

      - Analytics events based on AI engine

        - Object detection: Person/Face/Vehicle/License plate

        - Attributes: Vehicle(Type: car/bus/truck/motorcycle/bicycle)

        - Support DetectionShot

        - IVA: Virtual line(Crossing/Direction), Virtual area(Loitering/Intrusion/Enter/Exit)

      - Analytics events: Defocus detection, Motion detection, Tampering, Fog detection, Shock detection, Virtual area(Appear/Disappear)

        - Motion detection with 8 definable detection areas with 8 point polygonal zones, and minimum/maximum object size.

        - Audio Detection and classification of the following sound with NW I/O Box (External Device): Scream, Gunshot, Explosion, Crashing glass

      - Object auto tracking(Person/Vehicle), Target lock tracking based on AI engine

      - Alarm I/O: None (with NW I/O Box)

      - Alarm Triggers: Analytics, Network disconnect, Alarm input (with NW I/O Box)

      - Alarm Events: When alarm trigger occurred

        - File upload(image): e-mail/FTP

        - Notification: e-mail

        - Recording: SD/SDHC/SDXC or NAS recording at event triggers

        - PTZ preset

        - Handover

        - Alarm output (with NW I/O Box)

      - Water Removal: support(Spinning dry)

      - Auto Tracking: Object auto tracking(Person/Vehicle), Target lock tracking

      - Edge Storage: Micro SD/SDHC/SDXC 2slot 1TB

      - Memory: 4GB RAM, 512MB Flash

    - Video Streams

      - Video compression: H.265/H.264(Main/Baseline/High), MJPEG

      - Resolution

        - 16:9 aspect ratio: 1920 x 1080, 1280 x 720, 800 x 448, 640 x 360

        - 4:3 aspect ratio: 1280 x 960, 1024 x 768, 800 x 600, 640 x 480, 320x240

        - 5:4 aspect ratio: 1280 x 1024, 720 x 576

        - 3:2 aspect ratio: 720 x 480

      - Maximum frame rate

        - H.265/H.264: Max. 60fps/50fps(60Hz/50Hz)

        - MJPEG: Max. 30fps/25fps(60Hz/50Hz)

      - Smart Codec: Manual(5ea area), WiseStreamⅡ

      - Video Quality Adjustment

        - H.264/H.265: Target bitrate level control

        - MJPEG: Target bitrate level control

      - Bit rate control method

        - H.265 and H.264: CBR or VBR

        - MJPEG: VBR

      - Streaming Capability: Multiple streaming (Up to 10 profiles)

      - Simultaneous users (total): 20 maximum (unicast), 128 maximum (multicast)

      - Interoperability

        - ONVIF protocol: profiles S, G and T.

        - SUNAPI(HTTP API)

        - Wisenet Open Platform

    - Network

        - Connectivity: Metal shielded RJ-45(10/100BASE-T)

      - Protocol

        - IP v4 / v6, TCP, UDP

        - Configuration: DHCP, LLDP

        - Web service: HTTP, HTTPS

        - Network Service: ARP, Bonjour, DNS, ICMP, NTP, SNMP v1/v2c/v3(MIB-2), UPnP

        - Media: RTP, RTCP, RTSP

        - Unicast: SRTP

        - Multicast: IGMP

        - Notifications: FTP, SMTP

        - NTCIP

        - MQTT

      - DDNS - The camera shall support DDNS services offered by the manufacturer and others publicly available service offerings.

      - QoS – Layer 3 DSCP

      - Security features

        - User password protection

        - The device shall not provide a manufacture default password. Initial password setting shall be required to access the camera.

        - A minimal level of password complexity shall be required by the camera.

        - The camera shall not have a manufacture back-door password.

        - The manufacturer shall provide a tool that provides the ability to make password changes to multiple cameras at the same time.

        - IP address filtering – List of allowed or blocked IP addresses.

        - SRTP secured video communication for multicast.

        - Digest login authentication.

        - User access log.

        - 802.1X Authentication (EAP-TLS, EAP-LEAP)

        - Device Certificate (Hanwha Vision Root CA)

        - HTTPS(SSL/TLS) login authentication and secured video communication with TLS v1.3

        - Disk encryption on SD card recording (AES-256)

        - File encryption on local disk recording via web browser (AES-256)

        - Mutual Authentication (Client Authentication) for secure communication

        - Secure OS and Security Boot

      - Discovery – The manufacturer shall offer a discovery program to identify all devices of them on the network.

      - Configuration – The manufacturer shall offer a configuration program that remotely allows users to change settings on multiple cameras simultaneously.

      - Firmware upgrade – The manufacturer shall offer a program capable of upgrading multiple cameras at the same time (not requiring access to individual cameras).

      - Camera backup setting – The manufacturer shall provide a program that provides the ability to save multiple camera settings to a file and restore these camera settings if needed.

      - Reporting – The manufacturer shall provide a tool that can generate a report including thumbnail view, MAC address, IP address, serial number and other camera settings.

    - Electrical

      - Power Input Voltage: PoE+(IEEE802.3at, Class4)

      - Power Consumption: Typical 24W, Max 25.5W

    - Mechanical And Environmental

      - Color / Material: Body(White / Aluminum), Head(Black / Polycarbonate), Hard-coated dome

      - RAL Code: White(RAL9003), Black(RAL9005)

      - Dimensions (W x H): ø158x293.3mm(6.22x11.55")

      - Weight: 3.2kg (7.05lb)

      - Temperature / Humidity

        - Operating: '-40°C~+50°C(-40°F ~ +122°F) / +74°C(+165°F) (MAX) based on NEMA-TS 2(2.2.7) * Start up should be done at above -30°C / 0~95% RH(Non-condensing)

        - Storage: -50°C ~ +60°C(-58°F ~ +140°F) / 0~95% RH(Non-condensing)

      - Certification: IP66, IK10, NEMA4X, NEMA-TS 2(2.2.8, 2.2.9)

    - DORI

      - Detect (25PPM/ 8PPF): Wide: 59.5m(195.3ft) / Tele: 2340.4m(7678.4ft)

      - Observe (63PPM/ 19PPF): Wide: 23.8m(78.1ft) / Tele: 936.2m(3071.4ft)

      - Recognize (125PPM/ 38PPF): Wide: 11.9m(39.1ft) / Tele: 468.1m(1535.7ft)

      - Identify (250PPM/ 76PPF): Wide: 6.0m(19.5ft) / Tele: 234.0m(767.8ft)

    - EMC & Safety

      - FCC

        - 47 CFR Part 15 Subpart B(ANSI C63.4a-2017 Class A)

        - ICES-003 Issue 7(ANSI C63.4a-2017 Class A)

      - CE

        - EMC-Directive 2014/30/EU(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

      - UKCA

        - Regulations 2016(EN 55032:2015/A11:2020 Class A, EN 50130-4:2011)

      - RCM AS/NZS CISPR 32:2015 AMD 1:2020 Class A

      - VCCI CISPR 32:2016 Class A

      - KS C 9832 Class A, KS C9835

      - UL 62368-1, CSA C22.2 62638-1:19, IEC 62471

      - CE EN IEC 63000:2018 (hazardous substances)

END OF SECTION